5.1. Overview of environmental aspects, compliance with EU and national legislation, issues during the implementation of the Action and measures taken.

Effects on protected areas and their protection zones.

The proposed activity is located in an area where, according to the Nature and Landscape Protection Act, the I. (general) nature conservation status applies. In some sections, the railway line interferes with several areas protected under the aforementioned act.

No proposed extensions of the current railway body are assumed in none of these sections within the modernization, therefore no significant impact on the subjects of protection is expected.

The evaluated activity does not interfere with any protected tree in terms of the aforementioned Act; respectively not any other tree is located in its immediate vicinity.

Impact on Natura 2000 sites

According to information of the Administration of Záhorie Protected Landscape Area (Valachovic, 2016) there are no significant habitats in the affected areas, which could be affected by modernization works.

Since the railway line in its current alignment already crosses those protected areas, no directional changes are proposed in those sections and at the intersections there are no significant habitats, the proposed modernization is not expected to significantly influence these locations.

During the realization of the construction, a temporary construction access road will be used on both sides of the railroad body which in the vicinity of protected areas will be limited to unilateral access to reduce negative impacts on valuable biotopes.

During the reconstruction of the existing bridge, the realization of the new bridge over the Porecký stream in the National Nature Reserve Abrod will require a temporary takeover of land for the location of technology for bridge construction. This will lead to the degradation of protected habitats, so the takeover will be minimized to only necessary requirements.

For activities that affect habitats of national or European importance and which may cause them to be damaged or destroyed, it in terms of Article (6) (2) of the Act on Nature and Landscape Protection requires the consent of the Nature Protection Authority.

The consent of the Nature Protection Authority is also necessary in the case of wetlands (Section 6 (4) of the Nature and Landscape Protection Act).

Effects on locations in Ramsare: we do not expect a negative impact on the location water protection of the site.

Mitigation measures

Modernization of the railway line section Devínska Nová Ves - Kúty - state border SK/CZ will require takeover of agricultural and forest land.

The proposed action will as a result of the proposed line re-routing interfere with agricultural land and at the same time it will be necessary to temporarily remove the agricultural land for the purposes of placement of deposition area, handling areas, etc.

The proposed activity will also as a result of the proposed line re-routing interfere with forest land. The land concerned will be required in terms of Article (7) (1) (a) of the Act No. 326/2005 Coll. on Forests as amended of the National Council of the Slovak Republic to exclude from the fulfillment of the functions of forests by means of a permanent change in the nature of the land. As compensation for the loss of non-productive functions of the forest, in terms of Article 8 (2) (e) and Article 9 of the above mentioned act the financial compensation is determined. Since the evaluated construction has the character of railroad construction, including its components, in terms of Article (9) (7) (c) of the above mentioned act is exempted from charges.

In some parts of the affected area a tree cut will be necessary, which will be specified in the next stages of project documentation. Pursuant to the Act on Nature and Landscape Protection and in accordance with its Implementing Decree of the Ministry of Environment of the Slovak Republic No. 24/2003 Coll. as amended, the social value will be determined for the necessary cut-offs. Subsequently, a financial compensation of the given social value or an adequate planting will be determined by the Nature and Landscape Protection Authority.

In the case of habitats of national or European importance and their degradation, an appropriate revitalization action will have to be taken or a financial compensation will have to be paid up to the level of the social value of the affected habitats (within the terms of the above mentioned legislation).